STATE OF WISCONSIN

DEPARTMENT OF TRANSPORTATION

BUREAU OF AERONAUTICS

MODIFICATION OF CONSTRUCTION STANDARDS

MODIFICATION NO.: 2              PROJECT NO.:  AIP 3-55-0049-03

AIRPORT:  Lakeland Airport, Minocqua, Wisconsin CONTRACT NO.: 1

PROJECT DESCRIPTION:  Reconstruct and widen Rwy. 18/36; install HIRL.

REQUEST FOR MODIFICATION OF CONSTRUCTION STANDARDS:  This request for modification of construction standards is as follows:

A.	Description of the type of modification: This is a construction modification for a spec that has been routinely and successfully used on Wisconsin airports from the early 1980s thru the early 1990's.

B.	List of standards requiring modification: P-208 will be substituted with a "cold-mix asphalt pavement recyling" material designated on this project as "FULL DEPTH RECLAMATION.

C.	Description of the proposed modification:  Approximately 5.5" of existing asphalt pavement and 4.5" of existing crushed aggregate base course along with 2" of virgin P-208 will be ripped or pulverized then processed in-place or removed and processed in a central plant.  The material are mixed cold and will be used as a base course.  These materials will be reworked by means of mechanical stabilization.  Asphalt cement WILL NOT be added.  A hot mix wearing surface will be constructed to provide a water and abrasion resistant surface.

D.	Description of unique local conditions requiring the modification of standards, revised procedure, or new material: The existing pavement structure can be recylced to provide a high quality base course at much less cost then discarding the existing pavement structure and placing only virgin base course.  The current limited funding conditions under the Block Grant program require that we reuse resources to the maximum extent possible.

E.	A discussion of how proposed modification will provide an economical and feasible alternative to the FAA standard: The proposed substitution of P-208 with cold-mix asphalt recylcing has the following benefits:

a.	Existing base course deficiencies can be corrected.

b.	Transportation costs are reduced because recycling reduces the need for hauling new material and disposing of old material.

c.	Production costs are recuded because the nix is not heated, thus emission control systems and costly paving plants are not needed.

d.	The cold mix of asphalt pavement and crushed aggregate base course provides or exceeds the engineering properties of a 100% virgin crushed aggregate base course.

Quality control of the cold-mix is a limitation, however that can be overcomed by specifying 100% compaction based on a test section.

In addition to lower bids due to utilizing recylced materials, lower bids can be achieved because the contractors are quite familiar with the spec.  The contractors can execute the work with confidence.

- A discussion on how modification will provide an acceptable level of safety and service life: Only one failure has occured using the cold-mix asphalt pavement spec in liew of all virgin P-208.  That occurred over 14 years ago.  The failure was due to excessive voids in the pulverized asphalt.  This was overcome by requiring at least 50% underlying crush aggregate base course (or virgin P-208) be blended to fill the voids in the pulverized asphalt pavement.  No failures have been detected since the update with some pavements being 13 years old.

A discussion of how proposed modification was successfully used under similar conditions:  The proposed modifications reflect tried and proven standards that have been used on a number of airport jobs in Wisconsin since the early 1980s.

The first-time use of this modification on a federally funded project occurred in the early 1980's.  We estimate that the recycled cold-mix has been used on approximately 20 airport projects since then with excellent results.  A number of projects were desgnedfederally funded projects where the pavement was designed for aircraft less than 12,500# also utilized the new WisDOT-Hwy spec.   Attached is the cover letter from 1995 which conveyed the supporting documentation ensuing Mr. Dick Worch's and Mr. Dick Root's verbal understanding to proceed with these modifications.

ASSURANCE STATEMENT:

This is to assure that the proposed modification of construction standards will provide a product that will meet or exceed FAA standards for acceptance and that the finished product will perform for its intended design life based on historical data:

ENGINEER

Name of Firm:  BECHER-HOPPE ASSOCIATES, INC

BUREAU OF AERONAUTICS

Date

Thomas DeWinter, P.E., Chief

Airport Construction Standards

Date

Mark Arnold, P.E., Chief

Airport Engineering Section

APPROVAL:

FEDERAL AVIATION ADMINISTRATION

Approved by:                                            Title                        Date

Note:  A copy of this approval will be attached to either the "SPONSOR CERTIFICATION FOR PROJECT PLANS AND SPECIFICATIONS" or "SPONSOR CERTIFICATION FOR CONSTRUCTION PROJECT FINAL ACCEPTANCE" as appropriate.