STATE OF WISCONSIN

DEPARTMENT OF TRANSPORTATION

BUREAU OF AERONAUTICS

MODIFICATION OF CONSTRUCTION STANDARDS

MODIFICATION NO.:             1                   	PROJECT NO.:  3-55-0038-05

AIRPORT:   Manitowoc County                       	CONTRACT NO.:     1

PROJECT DESCRIPTION:   Reconstruction and expansion of the south general aviation apron

REQUEST FOR MODIFICATION OF CONSTRUCTION STANDARDS:  This request for modification of construction standards to accommodate unique local conditions is as follows:

- List of standards requiring modification and a discussion of why the standard cannot be met:  Section 152-2.2 f Compaction requirements. Existing cohesive soils and a high moisture content are making it difficult to compact the area.  The County has attempted to meet the density requirement (achieving densities of 78-95%).  The material could be removed and replaced with desirable material, but it would be cost prohibitive to EBS the entire apron area.

B.  A description of the proposed modification: The area will be thoroughly disked and recompacted to achieve the maximum density.  To correct the undesirable properties of these soils, a geogrid membrane will be placed as a separation layer between the undesirable cohesive soils and the non-cohesive recycled bituminous material.

C. A discussion of how proposed modification will provide an economical and feasible alternative to the FAA standard: 	 The modification will save the cost of EBS for the entire area.  The use of geogrid for material separation is a technically desirable alternative to wherever cohesive subgrade susceptibility to moisture accumulation, softening or strength loss is a problem.  Geogrid will increase the subgrade strength.  The geogrid will inhibit shear failure in the recycled material placed on the grid, resulting in a considerable increase in soil ultimate strength and load deformation behavior in this material.

D.  A discussion of how proposed modification was successfully used under similar conditions:  The County has successfully used the geogrid membrane in areas with unstable/undesirable material.

ASSURANCE STATEMENT:

This is to assure that the proposed modification of construction standards will provide a product that will meet or exceed FAA standards for acceptance and that the finished product will perform for its intended design life based on historical data:

ENGINEER

Name of Firm

by:                                                    Title                      Date

BUREAU OF AERONAUTICS

Date

C. M. Gelderman, P.E.

Specifications Development Chief

Date

Keith F. Richardson, P.E., Chief

Airport Engineering Section

APPROVAL:

FEDERAL AVIATION ADMINISTRATION

Approved by:                                            Title                        Date

Note:  A copy of this approval will be attached to either the "SPONSOR CERTIFICATION FOR PROJECT PLANS AND SPECIFICATIONS" or "SPONSOR CERTIFICATION FOR CONSTRUCTION PROJECT FINAL ACCEPTANCE" as appropriate.

318cdev.dot/r.02/22/95