STANDARD SPECIAL PROVISION P501-005

PORTLAND CEMENT CONCRETE PAVEMENT

Portland Cement Concrete Pavement. Replace the second paragraph of subsection 501-5.1 ACCEPTANCE SAMPLING AND TESTING with the following:

A lot shall consist of approximately [x] square yards ([y] square meters).  The lot's location will be determined based on the order of placement of the successive pours.  When approximately [x] square yards ([y] square meters) are placed, Lot 1 shall end and Lot 2 shall begin.  As production continues similar subdivision shall occur.

NOTE TO SPECIFIER

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

For small projects or projects where extremely small daily pours are anticipated, the Specifier may wish to replace the standard "daily" lot language with a lot definition based strictly on area in square yards (square meters).  Under this special provision there is no minimum requirement to make beams every day.  This special provision requires beams to be made in accordance with the random sampling of ASTM D 3665.  The Specifier shall specify the lot size for a project based on the total quantity and the expected production rate.  The lot size should not exceed 2,000 cubic yards (1 530 cubic meters) of in-place concrete.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

Add the following language at the end of paragraph 501-5.1(a.)(1):

A minimum daily sampling requirement to make flexural strength specimens (beams) does not apply.  The point (location) of beam production is strictly dependent on random sampling techniques based on the lot size, regardless of the daily concrete produced.