| ## MAINTENANCE TREATMENT REPORT Wisconsin Department of Transportation DT1400 6/2026 s.66.302 Wis. Stats. |  |  |  | # (see codes and instructions on back) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| County | Municipality |  |  | Measurements (check only one) Feet Tenth of a Mile Hundredth of a Mile |  | Date (m/d/yy) | Sheet of |  |
| Maintenance Treatment Type Codes (descriptions on back) |  |  |  |  |  |  |  |  |
| 1 = Crack Sealing – Rout and Seal |  | 4 = Wedge Patch | 8 = Lane-Shoulder Drop-Off Repair |  | 12 = Add Stone and Regrade |  |  |
| 2 = Crack Sealing – Overband |  | 5 = Sealcoating – Slurry Seal | 9 = Joint Sealing – Overband |  | 13 = Grade |  |  |
| 3 = Patching |  | 6 = Sealcoating – Micro Surfacing | 10 = Spot Grade |  | 14 = Spot Repair |  |  |
|  |  | 7 = Sealcoating – Chip Seal | 11 = Add Stone and Regrade In Spot Locations |  |  |  |  |

Multiple Treatment Types (for each road multiple types may be entered)

| On Road / Street | At Road / Street | At Offset | ###### Toward Road / Street | Toward Offset | ###### Surface |  | Treatment Type(s) | Treatment Year | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Type | Width |  |  |  |
| (SAMPLE) Hickory Lane | Dunning Road | 0 | #### Dead End | 0 | 55 | 24′ | 1,7 | 2012 |  |

| ## Instructions – Maintenance Treatment Report |  | Wisconsin Department of Transportation |
| --- | --- | --- |
| SURFACE TYPE 20 = Waterway 25 = Unimproved Road 30 = Graded and Drained Earth Road 35 = Gravel Road (not oil & gravel) 40 = <1" Wearing Surface 45 = Cold Mix Asphalt Pavement on Concrete 50 = Cold Mix Resurf on Asphalt Pvmt Surface + Base <7" 52 = Cold Mix Resurf on Asphalt Pvmt Surface + Base >7" 55 = Cold Mix Asphalt Pvmt (CMAC) Surface + Base <7" 57 = Cold Mix Asphalt Pvmt (CMAC) Surface + Base >7" 60 = Hot Mix Asphalt Pavement on Concrete (HMAC on PCC) 65 = Hot Mix Resurfacing (overlay) on Asphalt Pavement 70 = Hot Mix Asphalt Pavement (HMAC) 75 = Concrete Pavement (PCC) 80 = Brick or Block Pavement NOTE - A sealcoat pavement (Type 40) is a sealcoat surface over gravel or stone base. - Pavement sections with more than one inch of cold mix or hot mix material are not sealcoat pavements. These pavements (more than 1" of cold mix or hot mix) are classified as cold mix or hot mix pavements (categories 45–70), even if a sealcoat is present on these sections | MAINTENANCE TREATMENT TYPE CODES AND DESCRIPTIONS 1 = Crack Sealing – Rout and Seal – A crack sealing treatment where crack routing allows the sealant to remain below the pavement surface. Rout and seal may be a preferred method on cracks with higher degrees of movement, such as transverse cracks. The rout and seal method creates a crack seal reservoir that promotes adhesion between the sealant and the pavement by preparing the pavement edge and increasing surface area for adhesion. Having sealant below the surface protects it from traffic and snowplows. Treatment for surface types: 75, 70, 65, 60, 57, 55, 52, 50, 45. 2 = Crack Sealing – Overband – Usually a combination of asphalt cement and fibers used for crack sealing treatment in which sealant is poured/injected into the crack and an “overband” remains above the filled crack on the pavement surface. Treatment for surface types: 75, 70, 65, 60, 57, 55, 52, 50, 45. 3 = Patching – Placement of a repair material to replace a localized defect in the pavement surface. Treatment for surface types: 75, 70, 65, 60, 57, 55, 52, 50, 45, 40. 4 = Wedge Patch – Typical use is pavement edges strengthened or repaired with wedges of asphalt. Treatment for surface types: 70, 65, 60, 57, 55, 52, 50, 45, 40. 5 = Sealcoating – Slurry Seal – A mixture of asphalt, mineral aggregate, water, and specified additives, proportioned, mixed, and uniformly spread over a prepared surface. Treatment for surface types: 70, 65, 60, 57, 55, 52, 50, 45. 6 = Sealcoating – Micro Surfacing – A polymer-modified asphalt and aggregate mixture of dense-graded aggregate, asphalt emulsion, water and mineral fillers, spread over a prepared surface. Treatment for surface types: 70, 65, 60, 57, 55, 52, 50, 45. 7 = Sealcoating – Chip Seal – A surface treatment in which pavement is sprayed with asphalt followed by an aggregate cover and rolled. Treatment for surface types: 70, 65, 60, 57, 55, 52, 50, 45. 8 = Lane-Shoulder Drop-Off Repair – A correction in the elevation between the pavement edge and the shoulder. Treatment for surface types: 75, 70, 65, 60, 57, 55, 52, 50, 45, 40. 9 = Joint Sealing – Sealant material used to seal joints and openings to prevent water and incompressible materials from entering the joint through the joint. (See rout and seal and overband methods above). Treatment for surface types: 75. 10 = Spot Grade – Smoothing out areas of the roadbed with earthmoving equipment. Treatment for surface types: 35, 30, 25. 11 = Add Stone and Regrade In Spot Locations – Adding stone in specific areas and reshape or regrade to restore roadbed. Treatment for surface types: 35. 12 = Add Stone and Regrade – Adding stone and reshape or regrade to restore roadbed. Treatment for surface types: 35. 13 = Grade –Smoothing out the roadbed with earthmoving equipment. Treatment for surface types: 35, 30, 25. 14 = Spot Repair – Brick and Block repair to replace broken bricks, poor drainage and/or open joints. Treatment for surface types: 80. NOTE: For each road, MULTIPLE treatment types may be used. |
| SURFACE WIDTH Face of curb to face of curb, painted edge line to painted edge line OR inside edge of shoulder to inside edge of shoulder |  |  |
| NOTE - Culvert information will be maintained on the Route Comment webpage. - Ditch information will be maintained on the Route Comment webpage. - Curb information is currently a collected attribute in WISLR. - Please use the form DT1402 Construction Report – Certification Year for updates. |  |  |
| ELECTRONIC VERSION Microsoft Word form available by email: WISLRinfo@dot.wi.gov OR Sign up for Wisconsin Information System for Local Roads (WISLR) web site access: wisconsindot.gov/Pages/doing-bus/local-gov/wislr/default.aspx |  |  |