GEOMETRIC DATA

9.	Roadway Width (ft.) ______________________________

10.	Deck Width (ft.) __________________________________

11.	Culvert Barrel Length (ft.) __________________________

12.	Height, Clear Opening of Culvert (ft.) _________________

13.	Width, Clear Opening of Culvert (ft.) _________________

13.	Structure Length (ft.) _____________________________

14.	# of Lanes on Structure ___________________________

APPROACH DATA

15.	   Approach Roadway Width (ft.) ___________________

CAPACITY DATA

16.    Design HS/HL ________________________________

17.	Inventory HS/RF_________________________________

18.	Operating HS/RF ________________________________

19.	Maximum Vehicle Weight (Kips) ____________________

20.	Load Rating Basis – LFR/LRFR

21.	Load Governing Member -  Slab    _______________

22.	Overburden Type/Material:  ________________________

23.	Overburden Depth (inches) _________________________

HYDRAULIC DATA

24.    Design Flood Frequency in Years ___________________

25.    Design Discharge (ft3/sec) _________________________

26.	Max. Velocity (ft. /sec.) ____________________________

27.	Drainage Area (sq. miles) __________________________

28.	High Water Elev. (ft.) _____________________________

29.	Scour Critical Code:  _____________________________

STRUCTURE SERVICE DATA

30.	Hwy. On Detour Length (Miles) ____________________

31.	Type Service On

Highway

Other___________________

32.    Type Service Under

Waterway

Pedestrian

Cattle Pass

PLANNING DATA

33.	Functional Classification:

Interstate Rural (1)            Interstate-Urban (11)

Other Art-Rural (2)             Other Art.-Urban (14)

Local-Rural (9)	                Local Urban (19)

34.	Traffic Direction On:    None     1-Way     2-Way

CONSTRUCTION DATA

35.		Plans Completed	YR _____ MO _____ DAY_______

36.		Letting Date	YR _____ MO _____ DAY_______

37.		Year Built	_______

38.		WORK Performed

New Structure	 Other _______________

39.		Designer:   ______________________

40.		Fabricator:   ______________________

41.		General Contractor:   ______________________

42.		Project ID:            -      -

43.		Cost:  ____________

PRIMARY ROUTE(S)

44.	Enter All Route numbers IH, USH, STH, or CTH numbers or Town Road carried by the structure.

___________________________________________

45.	Direction:		   North   	  East

South	  West

46.	Designation:	   Mainline	   Other _______

47.	Inventory Route:	   On NHS	  Not on NHS

48.	Location: ____________________

49.	Enter Feature that goes under/through structure.  Waterway name, cattle crossing or pedestrian.

_______________________________________________

50.	Designation:    Waterway 	  Pedestrian

SPAN DATA

| 51. | Span # | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 52. | Length, ft. |  |  |  |  |  |
| 53. | Configuration |  |  |  |  |  |
| 54. | Material |  |  |  |  |  |

TRAFFIC DATA

55.	ADT __________________________

56.	ADT-Year ______________________

General Instructions and Help

Box 1. 	 Enter the assigned ID for the structure.  Structure ID’s can be obtained by contacting the Regional Inspection Program Manager.

Box 2. 	 Enter the region where the structure is located.  If located on a boundary between two Regions, enter the Region associated with the structure ID.

Box 3. 	 Enter the county where the structure is located.  Note that the county must match the structure ID value.

Box 4/5.	 Enter the maintainer/owner of the structure.  In some instances these codes will be different, but this is rare.

Box 6. 	 Enter the municipality where the structure is located.

Box 7/8.  	Enter the Latitude and Longitude of the structure using the following format:  DDMMSS.S where DD is degrees, MM is minutes, and SS.S is seconds.

Box 9.  	Code the Roadway Width (Item 51) per the guidelines in the Recording and Coding Guide (RCG) for Bridge Roadway Width.  The RCG can be found at this location:  https://www.fhwa.dot.gov/bridge/mtguide.pdf .

Box 10.	Code the Deck Width (Item 52) per the guidelines in the RCG.

Box 11.	If structure configuration is a box culvert, a sectional plate pipe arch, or other similar structure, code the barrel length in this field.  This includes the total length (along the flow line) of the structure, excluding aprons.

Box 12/13.	If box 11 is filled out, complete both 12 and 13 as well for the structure in question.  Distances are clear between walls or floor/ceiling.  If a multiple cell box culvert, only code the width of the smallest cell.

Box 14.	Using the definition in the RCG for Item 28 – Lanes On Structure.

Box 15.	The definition for Approach Pavement Width (Item 32 of the RCG) shall be used to fill out this item.

Box 16.	Enter the vehicle for which the structure was designed, if known.

Box 17/18. Enter the inventory and operating load ratings for the structure in question.  For LFR, the entry shall be in HS values.  For LRFR, the values must be RF.

Box 19.	Record the maximum vehicle weight allowed for the Wisconsin Standard Permit Vehicle.  The maximum allowed value is 250 kips.

Box 20.	Indicate the load rating methodology used in the load rating assessment.

Box 21.	Indicate the controlling member for the load rating.

Box 22/23. Indicate the type of overlay, and the thickness of the overlay.  For filled structure, indicate the total amount of fill but for type simply indicate the top surface.

Box 24-28.  If hydraulic calculations were used to size the structure, provide the pertinent information in these fields.

Box 29.	Per item 113 of the RCG, enter the scour critical code for the structure.  If not over water, enter code N.  If unknown, use U.  If box culvert, enter 5.

Box 30.	Per item 19 of the RCG, enter the detour length to the nearest mile.

Box 31.	Indicate the services carried by the structure.  Usually this is highway, but there may also be pedestrian or railroad.  Entry all that are valid.

Box 32.	Enter the most appropriate service type going under/thru the structure.

Box 33.	Enter the functional classification of the roadway being carried by the structure per item 26 of the RCG.  In general, if the structure isn’t located in a designated urban area, it should be coded rural.  http://wisconsindot.gov/Pages/projects/data-plan/plan-res/function.aspx  is a link with more information.

Box 34.	Per item 102 of the RCG, enter the direction of traffic.

Box 35.	Indicate the date the plans were completed, if known.

Box 36.	Enter the letting date of the project, if known.

Box 37.	Enter the year the project was completed or will be completed.

Box 38.  	Enter the type of work that was done on the structure.  Common work types include:

| Code | Work Name |  | Code | Work Name |  | Code | Work Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NEW STRUCTURE |  | 58 | OVERLAY - CONCRETE - NEW JOINTS |  | 9 | LENGTHEN STRUCTURE |
| 6 | NEW DECK |  | 20 | OVERLAY - CONCRETE - NEW RAIL & JOINTS |  | 43 | MISC REPAIRS |
| 68 | NEW DECK/WIDENING |  | 78 | OVERLAY - PMA |  | 7 | PAINTING |
| 49 | NEW EXPANSION JOINTS |  | 81 | OVERLAY - POLYESTER |  | 75 | PAINTING - SPOT |
| 52 | NEW JOINTS & BRGS |  | 77 | OVERLAY - POLYMER |  | 28 | REPAIR DECK |
| 11 | NEW RAILING |  | 21 | OVERLAY - BITUMINOUS |  | 14 | REPAIR SUBSTRUCTURE |
| 46 | NEW SUBSTRUCTURE |  | 3 | OVERLAY - CONCRETE |  | 10 | REPAIR SUPERSTRUCTURE |
| 8 | NEW SUPERSTRUCTURE |  | 2 | WIDEN STRUCTURE |  | 72 | REPAIR/REPLACE WINGS |

Box 39.	Enter the designer of record for the structure.

Box 40.	Enter the primary fabricator of the sign/signal structure.

Box 41.	Indicate the contractor who installed or will install the structure in the field.

Box 42.	Enter the construction project ID.

Box 43.	If project is complete, enter the cost.

Box 44.  	Enter the primary route traveling on the structure.  If multiple concurrent routes exist, name them all.  But list which is primary by the following hierarchy:  IH, USH, STH, CTH, then Local Road.  If multiple of the same type exist, use lower number.  For example, say the sign spans IH 39 NB and IH 90 WB.  The primary route would be IH 39 NB.

Box 45.	Enter the directions that the roadway carries.

Box 46.  	Enter the function the primary roadway on performs (i.e. Mainline, Ramp, Frontage Road, etc.)

Box 47.  	Indicate if the primary roadway on carries the National Highway System.  All IH and USH carry the system, but so do some other roads.  More information on this can be found on this website:  http://www.fhwa.dot.gov/planning/national_highway_system/nhs_maps/wisconsin/index.cfm

Box 48.	Indicate the location using guidance from the RCG, Item 9.

Box 49.	Indicate the name of the feature going under/thru the structure.

Box 50.  	Indicate the type of service going under/thru the structure.  Usually this is a waterway, pedestrian pass, or cattle pass.

Box 51. 	Enter the Span Numbers.  Each cell of a box culvert is considered a span in this instance.

Box 52.	Corresponding to the span number entered in box 51, enter the clear span length of the span/cell in Feet.

Box 53.	Corresponding to the span number entered in box 51, enter the configuration using one of the codes below:

| Code | Span Configuration |  | Code | Span Configuration |  | Code | Span Configuration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 04 | FLAT SLAB |  | 70 | BOX CULVERT |  | 40 | RIGID FRAME |
| 05 | OTHER SLAB |  | 72 | PRECAST CULVERT |  | 41 | OTHER FRAME |
| 10 | DECK GIRDER |  | 92 | SECTIONAL PLATE PIPE ARCH |  | 52 | FILLED ARCH |
| 11 | THRU GIRD/FL SYS |  | 15 | BOX SECTIONS |  | 54 | PRECAST ARCH |
| 19 | OTHER GIRDER |  | 16 | CHANNEL |  | 59 | OTHER ARCH |

Box 54.	Corresponding to the span number entered in box 51, enter the material using one of the codes below:

| Code | Material |  | Code | Material |  | Code | Material |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | CONCRETE |  | 05 | PREST CONCRETE |  | 09 | ALUMINUM |
| 02 | CONT CONCRETE |  | 06 | CONT PREST CONC |  | 10 | WROUGHT IRON |
| 03 | STEEL |  | 07 | TIMBER |  | 15 | GALV STEEL |
| 04 | CONT STEEL |  | 08 | MASONRY |  | 16 | PRECAST CONC |

Box 55.	Enter the ADT for the primary roadway carried.  For more information on traffic counts, look into this worksheet:

http://wisconsindot.gov/Pages/projects/data-plan/traf-counts/default.aspx

Box 56.	Enter the year the ADT was measured/estimated.