# Bearing Repair, Item SPV.0060.xx.

A  Description

This special provision describes removing the bearings at substructures noted in the plans, blast cleaning and priming the bearings, furnishing and placing shims, resetting the bearings, and furnishing and placing new anchor bolts.

B  (Vacant)

C  Construction

Clean the bearings to a near white finish. Prime the bearings with one coat of organic zinc primer.

D  Measurement

The department will measure Bearing Repair as each individual bearing acceptably completed.

E  Payment

The department will pay for the measured quantity at the contract unit price under the following bid item:

| ITEM NUMBER | DESCRIPTION | UNIT |
| --- | --- | --- |
| SPV.0060.xx | Bearing Repair | Each |

Payment for Bearing Repair is full compensation for removing, cleaning, painting, and resetting bearings; and furnishing, painting, and placing shim plates and anchor bolts.