# Cleaning and Painting Bearings, Item SPV.0060.xx.

A  Description

This special provision describes cleaning and painting the existing steel bearings on structures conforming to standard spec 517 and as directed by the engineer.

B  Materials

Furnish a complete coating system from the department’s Painting Epoxy System Structure approved product list.  Use the same coating system for all repairs due to handling, shipping, and erecting; and for all other uncoated areas.

The color of epoxy shall be XXXXXXX and the urethane coating material shall match the color number shown on the plans conforming to AMS Standard 595A.

Supply the engineer with the product data sheets before any coating is applied.  The product data sheets shall indicate the mixing and thinning directions, the minimum drying time for shop or field applied coats, and the recommended procedures for coating galvanized bolts, nuts, and washers.

C  Construction

C.1  Surface Preparation

Clean areas of loose paint and rust by wire brushing, grinding, or other mechanical means. Sound paint does not need to be removed.  After clean up and storage of waste material, blast cleaning is allowed for only those areas where paint has been removed.  Shield adjacent painted areas during blast cleaning operations.  The blasting sand does not have to be collected.

Furnish containment methods as required to contain and collect waste material resulting from the preparation of painted steel surfaces for painting.  All clean up activities should minimize dust.  Store waste materials in hazardous waste containers provided by the department.  The department is responsible for the transport and disposal of the contained materials by the statewide hazardous waste contractor.

C.2  Coating Application

Apply paint in a neat, workmanlike manner, and conforming to the manufacturer’s instructions and recommendations. Paint application shall be brushed on.

D  Measurement

The department will measure Cleaning and Painting Bearings as each individual bearing acceptably completed.

E  Payment

The department will pay for the measured quantity at the contract unit price under the following bid item:

| ITEM NUMBER | DESCRIPTION | UNIT |
| --- | --- | --- |
| SPV.0060.xx | Cleaning and Painting Bearings | Each |

Payment for Cleaning and Painting Bearings is full compensation for preparing and cleaning the designated bearings; furnishing and applying the paint; cleaning up; and containing and collecting all waste materials.  (20210426)