Designer note: Update sections B and C below, as necessary for specific project requirements (i.e., WI DNR requirements, wildlife corridor requirements, etc.). Special provision is used to prevent animal entrapment and to provide animal passage in a riprap slope.  This special provision is intended to cover a single structure location. If multiple structure locations are present, add additional SPV numbers.
Infill Riprap (structure), Item SPV.0195.xx.
A  Description

This special provision describes furnishing and placing material to fill voids in riprap slopes per the details shown on the plans.

B  Materials

Furnish select crushed material according to standard spec 312. 
C  Construction

Place the infill material after the riprap placement has been completed. Place material such that voids in the finished surface are 1.5-inches or less in any dimension. Infill to the top of riprap.
D  Measurement
The department will measure Infill Riprap (structure) by the ton acceptably completed.
E  Payment

The department will pay for measured quantities at the contract unit price under the following bid item:

	ITEM NUMBER
	DESCRIPTION
	UNIT

	SPV.0195.xx
	Infill Riprap (structure)
	Ton


Payment for Infill Riprap (structure) is full compensation for providing, placing, and shaping the material. 

SPV.0195.xx (20260105)

