# Precast Panels for Soldier Pile Walls, Item SPV.0165.xx.

A  Description

This special provision describes fabricating, furnishing, transporting and erecting precast concrete panels for soldier pile walls.

B  Materials

B.1 General

Furnish precast panels that conform to the details shown on the plans.

Furnish steel reinforcement conforming to standard spec 505.

Furnish Grade A air-entrained concrete conforming to standard spec 501.

B.2 Plant Certification

Obtain all precast concrete panels from fabrication plants that comply with the department’s plant certification program in standard spec 106.3.3.3.1.

C  Construction

C.1 General

Prior to fabricating panels, verify that no field modifications were made to the top of footing elevations. Field modifications will require adjustments to the panel heights listed on the plans.

C.2 Placing Concrete

Handle and place the concrete conforming to standard spec 502.3.5.

C.3 Curing

Cure the concrete panels by any of the methods specified in standard spec 502.3.8.

C.4 Transportation, Storage, and Erection

Transport, handle, store, and erect precast panels in a way that prevents cracking or other damage to the panels.  Discard and replace units damaged by improper handling or storing.

D  Measurement

The department will measure Precast Panels for Soldier Pile Walls by the square foot acceptably completed.

E  Payment

The department will pay for the measured quantity at the contract unit price under the following bid item:

| ITEM NUMBER | DESCRIPTION | UNIT |
| --- | --- | --- |
| SPV.0165.xx | Precast Panels for Soldier Pile Walls | SF |

Payment for Precast Panels for Soldier Pile Walls is full compensation for furnishing all materials, including leveling blocks for fabrication; for all handling, hauling and erecting; and for discarding and replacing units damaged by improper handling or storage.  (20211104)