- Designer note: Update sections B and C below, as necessary for specific project requirements (i.e., WI DNR requirements, particular wildlife corridor requirements, etc.).

- Select Crushed Material for Travel Corridor, Item SPV.0195.xx.

A  Description

This special provision describes furnishing and placing select crushed material to fill voids to create a wildlife travel corridor.

B  Materials

Furnish select crushed material according to the pertinent requirements of standard spec 312. Material shall be clean and substantially free from material passing the No. 4 (4.75mm) sieve.

C  Construction

Place the material after the heavy riprap has been completed. Place material such that voids in the finished surface are three inches or less in any dimension.

D  Measurement

The department will measure Select Crushed Material for Travel Corridor by the ton acceptably completed.

E  Payment

The department will pay for measured quantities at the contract unit price under the following bid item:

| ITEM NUMBER | DESCRIPTION | UNIT |
| --- | --- | --- |
| SPV.0195.xx | Select Crushed Material for Travel Corridor | Ton |

Payment for Select Crushed Material for Travel Corridor is full compensation for providing, placing, and shaping the material.